​AMENII - Analytical solutions to calculate MEchanical response of an aquifer to Non-Isothermal Injection.

This tool calculates mechanical response (displacements and stress variation) to non-isothermal single-phase fluid injection into an aquifer with cylindrical geometry. It is based on the paper:

"De Simone, S., Carrera, J., & Gómez-Castro, B. M. (2017).  A practical solution to the mechanical perturbations induced by non-isothermal injection into a permeable medium. International Journal of Rock Mechanics and Mining Sciences, 91, 7-17."

The complete solutions, as well as the underlying hypothesis, the mathematical development and the validity, are presented in the abovementioned paper, which is also included in this folder.

The analytical solutions are implemented both in an excel spreadsheet and in a Python script. User can choose the favourite format. Both files adopt the same nomenclature and units than the paper, which are also provided in the file “Nomenclature”.

The excel spreadsheet is organized in the following way:

- First sheet is called "input data"​and must be filled-in with the parameters of the problem. In the current form it presents the same values than in the paper.​ User has to fill-in the orange cells, whereas the white ones are just calculation of key parameters and​their editing is blocked.​

- Then there are two sheets that calculate results vs space (for a fixed time) and​vs time (for a fixed radius), respectively. Modifications is not allowed in these sheets in order to ​protect them. User only needs to set the time of observation at the top of the sheet "calculation vs radius" and the radius of observation at the top of the sheet "calculation vs time". The spatial and temporal discretizations can also been modified, although the proposed spatial discretization is suggested, as it is automatically calculated according with the input geometrical parameters.

- The last two sheet present plots of results, but other plots can be generated from the results columns.

The Python script (based on Python 2.7) is organized as follows:

- Input parameters have to be set at the top of the script. In the current form it presents the same values than in the paper.​ User can choose if calculate results vs space (for a fixed time that should be indicated where asked) or vs time (for a fixed radius that should be indicated where asked). Both options can also be chosen.

- The following lines calculate the results. Spatial and temporal discretizations can be modified by the user, although the proposed spatial discretization is suggested, as it is automatically calculated according with the input parameters.

- The script automatically generates the desired plots, which can be modified.

For more details please contact Silvia De Simone (silviadesi@gmail.com).